OEM-DES-RFID-Lock

13.56 MHz OEM RFID Lock with CAN interface

Development Description

| iDTRONIC GmbH Ludwig-Reichling-Straße 4 67059 Ludwigshafen Germany/Deutschland Phone: +49 621 6690094-0 Fax: +49 621 6690094-9 E-Mail: info@idtronic.de Web: idtronic.de | Issue 0.1 – 09. October 2020 – Subject to alteration without prior notice. © Copyright iDTRONIC GmbH 2020 Printed in Germany |
| --- | --- |

Contents

1	Project Overview	3

1.1	Firmware Update	4

1.2	Function Description Key-Operation (Standard Operation Mode)	4

1.3	Function Description Key Teach-In Operation	4

1.4	Configuration of the RFID-Lock	4

1.5	Needed CAN-Bus Commands	4

1.5.1	Initialising the RFID-Lock	4

1.5.2	RFID Key Handling	5

1.5.3	Service Commands	5

1.6	Glossary	5

2	Electric Requirements	5

2.1	Power Supply with Overvoltage Protection	5

2.2	Current Consumption	5

2.3	CAN-Bus with Overvoltage Protection	5

2.4	Connector	5

3	RFID Antenna	6

4	Mechanics	6

5	Environmental Conditions	6

6	Suggestion	6

7	Technical Data	6

8	Revision History	7

# Project Overview

The complete device shall act as an RFID lock for a vehicle.

The RFID lock will be a DESFire keyfob. AES Encryption shall be used with the data stored onto the DESFire RFID tag.

Access information must be stored in a Trusted Flash Memory (e.g. TrustZone with ST MCUs). This information is:

PICC password (do we really need this?)

application number

application password

file number

file password for read access

file password for write access

password for 3DES CAN-Bus encrypted communication

Communication with the vehicle is done using a custom-specific CAN implementation.

Communication with the vehicle is done using a 3DES encryption.

## Firmware Update

It must be possible to update all firmwares (MCU + RFID) over the CAN-Bus.

## Function Description Key-Operation (Standard Operation Mode)

The RFID module detects the RFID keyfob.

The RFID module wakes up the MCU.

The MCU will switch the RFID module to standard operation.

The MCU will send commands to the RFID module to read the key number (4 Bytes) using the previously stored access data from DESFire RFID tag.

If a key number can be successfully read from the DESFire RFID tag, the MCU will send this number to the vehicle computer.

The MCU will switch the RFID module to low-power card detection mode.

The MCU will switch itself to low-power mode.

## Function Description Key Teach-In Operation

The vehicle computer sends a CAN-Bus telegram to the RFID-Lock with the key number (4 Bytes).

The MCU will store the key number onto the DESFire RFID tag:

PICCActivate

RATS

Create Application

Select Application

Set Application Access Password

Set File Read Password

Set File Write Password

Create File

Write key number (8 Bytes) into the file

After this, the RFID-Lock shall test if the data is correctly written onto the DESFire RFID tag. If the data is correct, the RFID-Lock will confirm to the vehicle computer.

## Configuration of the RFID-Lock

Before the RFID device can operate as Lock all access information must be configured by the vehicle computer (or other IT equipment).

## Needed CAN-Bus Commands

### Initialising the RFID-Lock

SetPICCPW (do we really need this?)

SetAppAccessInfo (Flags, AppNr, KeyNr, Key)

SetFileAccessInfo (Flags, FileNr, KeyNr, Key)

Set3DESKey

### RFID Key Handling

TeachInKey (write 4 Bytes into file using initialized information)

SendKeyNr (automatically by the RFID-Lock)

### Service Commands

TransferFirmware2RFIDModule

TransferFirmware2MCU

GetVersionInformation

## Glossary

MCU = MicroController Unit

RFID = Radio Frequency Identification

App = Folder on DESFire RFID tag

# Electric Requirements

## Power Supply with Overvoltage Protection

Requirement

The supply voltage for the reader is between 7.6 V and 13.8 V.

The supply circuitry shall be able to withstand +60 Vdc and reverse polarity.

In overvoltage situation, there is no need to keep the normal function intact.

The power circuitry in the RFID Lock can switch off in case of overvoltage or reverse polarity.

## Current Consumption

Requirement

Sleep: During sleep mode the current consumption has to be « 1 mA.

Active: During active mode the current consumption can be up to 120 mA.

RFID Module:	< 50 mA

STM32L:	62 µA/MHz

Low-Power Mode

RFID Module: unknown

STM32L:	3 µA

## CAN-Bus with Overvoltage Protection

The CAN-Bus shall be protected to withstand +60 Vdc.

In overvoltage situation, there is no need to keep the normal function intact.

## Connector

JWPF from JST (pls. see data sheets from manufacturer)

# RFID Antenna

Internal, depending on mounting position. TBD

External, with IP65 proof connector (e.g. SMA).

The antenna will be tuned for the low-power card detection mode + keyfob form factor.

# Mechanics

The maximum dimensions shall not exceed 100 × 90 ×30 mm.

# Environmental Conditions

Operating temperature range: -10°C to +50°C

Storage temperature range: -20°C to +60°C

IP64 or more. (potted electronics)

# Suggestion

CHK305 with NXPs low-power card detection mode. Additional command to start this mode and configure it (if needed/possible).

STM32L5 as MCU with project-specific software and CAN interface.

Additional CAN-Bus interface LTC2875

Power supply circuit with overvoltage shut-off (sourced from other supplier).

# Technical Data

| Electrical Specifications |  |
| --- | --- |
| Power Supply | 7.6 … 13.8 V |
| Power Consumption | < 120 mA, standby current < 1 mA (low power mode) |
| Operating Frequency | 13.56 MHz |
| Baudrate CAN-Bus | 125 kbit/s, 250 kbit/s, 500 kbit/s, 1 Mbit/s |
| Antenna | TBD |
| Reader IC | CL 663 |
| RF TX Speed | up to 848 kBd |
| Interfaces | CAN-Bus with custom-specific communication protocol |

| Environmental Conditions |  |
| --- | --- |
| Operating Temperature | -10 °C … +50 °C |
| Storage Temperature | -10 °C … +60 °C |
| Protection Class | IP64 or more |
| Humidity | up to 95 %, non condensing |
| MTBF | 200‘000 h |

| Applicable Standards |  |
| --- | --- |
| EMC | EN 301489-1:2012-04 (v1.9.21) EN 301489-3:2013-12 (V1.6.1) |
| Radio Regulation | EN 300330-1:2015-08 (V1.8.1) EN 300330-2:2015-08 (V1.6.1) |
| Safety | EC Guideline 2011/65/EU and amendment 2015/863 EN 50581:2012 (valid till 2024-07-07) EN 63000:2018 |
| RoHS 2 | EC Guideline 2011/65/EU |
| REACH | EU Guideline 1907/2006, updated by 2018/2005/EU |
| Certificates | FCC, CE |

Other functions and details to be continued and upgraded.

# Revision History

| Date | Version | Description |
| --- | --- | --- |
| 2020-10-09 | 0.1 | Initial Draft |